Kids Tracker Watch Instruction

Please read the instruction manual carefully before using in order to install properly and use instantly. For the watch’s color, please check the real product.

I. Before using
Check if the unit is corresponding to package and all accessories are included.

Install a SIM Card and choose GSM network. For details, please refer to the seller's advice. (Attention: Please shut down the watch before card installation)

SIM card terminals must have GPRS and Caller ID function.

Install an Apk / App into Smart-phone. For the Apk / App, please check with your seller.
Bar-code or QR code for users registration

II. Features
1. GPS+LBS dual ways locating

2. Remote control by smart phone
3. GPRS real-time locating, tracking and monitoring;

4. intercom
5. Health
6. Track back
7. Alarm
8. Safety zone
9. SOS emergency alarm
10.Remove alarm
11. Low-battery alarm
12.Remote shutting down
III. Watch keys function
  [image: image1.png]



USB port: Charging and data transportation

SOS: SOS alarm and answer phone calls

Vol- during phone conversation. Hold for more than 3 seconds it will dial pre-loaded phone number 1

Power key and hang up phone calls

Vol+ during phone conversation. Hold for more than 3 seconds it will dial pre-loaded phone number 2
Ⅳ
[image: image2.png]Ta O @
w3 21

I
[Ny )

2014, 12.01

4, &10000



 

Ⅴ.APP instruction
5.1 APP registration
Before log in, you will need to manually register the account/ID. You can either scan the bar code or manually put the ID, the user name should be no less than 4 characters (letters or numbers), the ID and user name is unique and unrepeatable. The other information, user can manually put in, password can be changed. The registration interface is like below:

[image: image3.jpg]eeeco tEEE =

Login
©® Account: 10830 v

() Password: eeeeee v

Forget password?




      [image: image4.jpg]o HEEGE T 16:27

&= register

%) Device ID: scan or ini(" Scan the DID

© Account: 4-16 characters

@ Nickname: 2-16 characters

& P _number: retrieve password

@ Password: 6-12 characters

&5

=3 M D nacovrnrd: eonfirm nacewnrd




5.2 Log-in
5.2.1 APP functions menu
After you finish the registration, put in the correct account name and password, click log-in, you will find the home interface like below:
[image: image5.jpg]eeeco HMEELE T 16:27 -

. —

\_‘O_/ Intercom

r”i Footprint 4% Settings

Geofence . Rewards




Functions Description:
⑴ walkie-talkie：by this function, you can have voice conversation between smart watch and app
Send voice message over app: when the recording is initiated, the app will automatically record and send the recordings to the watch, maximum 15 seconds. On the watch, short press sos button to play the recordings.

Watch recording: long press the power button to record, recordings will be sent when hands released, max. recording duration is 15 seconds. The app will receive an alert when the voice message is received(the timing before you can receive the message is depending on your network operator)
⑵Map: real-time location can be viewed on the watch

The watch and app mobile device location can be found on the map, press "postioning" the watch will start to locate your device for 3 mins, the uploading interval is 10 seconds, after 3 mins it will restore to the default working mode. On the map, you can find the watch postioning way real-time. If the position icon is red, it is GPS positioning mode, blue, is LBS position, green is wifi positioning.

⑶ Settings: you can set the parameters in this menu
[image: image6.jpg]Sos/The family number
"E' Voice

® 5 work mode Normal mode: 10m/t
@ No disturbing 00:00-00:00

SMS alerts setting >

Contacts =

O Language and time zone >





①School location period: the four sections is only for Monday till Friday, in the set period, the watch will initiate the GPS and retrieve the location, out of the period will not retrieve any location.

②Weekend location period: these 2 sections is only for Saturday and Sunday. In the set period, the watch will initiate the GPS and retrieve the location, out of the period will not retrieve any location.

③Do not disturb period: All calls will be silenced in these 3 periods

④SMS alert settings: alert when the watch is low power, SOS button is pressed, or take off the watch

⑤Kids contact:you can set 10 numbers to call in the list

⑥Restore to default working mode: when the default mode is activated, the GPS will be enabled 24 hours, the location retrieving interval is 10 mins

⑦Set SOS numbers: can set 3 numbers, in emergency, alert will be activated by press and holding the SOS button for 3 seconds. It will call the 3 set numbers in a loop. If not answered, calling will be stopped after 2 loops

⑷Safety zone: the min.fence radius is within 500 meters, if the watch user walked out of the range, you will receive an alert

⑸Love rewards: on the app, choose the red-heart numbers, click confirm it will send to the watch immediately, the watch screen will show the love and numbers

⑹Watch alarm: can set 3 alarms, you can set the alarm mode to your needs

⑺Find the watch: if the watch is not nearby, send this command, the watch will start to ring for 1 mins so that you can easily find it, press any button will stop the ringer

For other functions, you can find a brief description in the "My account-help" menu
5.2.2 My account:
[image: image7.jpg]account

0 Change the password

e Update

HOME




For more detailed description for all functions, you can refer to the “help” section

5.3 Watch alert description

The app device will receive the push alert message if the watch initiate the alert. You can click alert on your app device to enter the home screen, for detailed alert info, click the information center.
Ⅵ、Watches Work Pattern 
Three basic working pattern as below, normal mode and the power saving mode, real-time positioning mode.
The different working pattern will be within different upload interval,real-time positioning mode will be worked in setting up the mode of "the school localization period" and "the weekend localization period" . Except this situation , the customer can be according to their requirements to set up working pattern. 

Normal mode: upload interval of 600 seconds; 
Power saving mode: upload interval of 3600 seconds; 
Real-time positioning mode: upload interval to 10 seconds;
Ⅶ.Instructions

7.1 SOS Emergency Alarm

Long press the SOS button for 3 seconds, watch will circularly dial three SOS number two rounds. If no answer, device will keeping calling till two circles calling finished.Watch can also send SMS Alert to monitering nubmers (default off) and also show on the APP. 

7.2 Anti-lost Alarm

Watch strap with sensor switch, watch to wear at least 3 minutes, remove the strap will generate alarm, watch will send alarm messages to monitoring number(default closed), and send the APP alarm notice.

7.3 Low power Alarm

Watch will send SMS Alert (default off) to monitering numbers and also send alarm notification to the app when battery power is lower than 20%. 

7.4 Health Care

7.4.1 Pedometer

Watch is equipped with 3D sensor which will calculate the user step when walking or running, the user can know your movements through the APP side.

7.4.2 Sleep Monitering

According to watch rotating frequency and amplitude to determine the user's quality of sleep, default detection time from 10:00 p.m. to 7:00 in the morning the following day, user can check related data through the APP.

7.4.3 Sport Function

Computing by step number, step pitch and weight, step pitch and weight can be set up.

7.5 Intercom System

Long press the power button to start recording, after releasing the power button to send the voice message to the APP.

If watch got voice messages from APP, there would be notification on the display. Press SOS button to play the voice massages.

7.6 Voice Monitering

Watch must preset monitering numbers, when given order command from watch user, device will make one way call to monitering numbers.

7.7 Power Off

Power off is permitted without SIM card installed or remote controlled by APP and no power.

7.8 Family Number

Watch has two affection dial key, long press to dail, the family number 1 corresponding sos1, family number 2 corresponding to the family number 2.

7.9 Contact List

App side preset 10 Numbers which permit to call the watch to implement two-way conversation.



Ⅷ. Troubleshooting 
When turning on the watch first time, App within off -line after landing. Please check: 
i. Check whether SIM card is installed properly and whether the screen display is normal. please follow the reference pictures.
ii. Check whether SIM card is fees owned, whether data traffic opened and whether signal is normal.
iii. Check whether the GPRS service is canceled.
iv. Consult your supplier whether the parameters of the terminal server has been set,and check the relevant parameters. The customer can use their mobile phone text commands sent to the watch phone card: pw, 123456, ts# .The watch will be sent parameters to the Mobile phone. 

[image: image8.png]



3   





4





1











5





2








GSM、GPS、Battery


loving heart 、voice message





Hint of remove watch 


Pedometer（Closed th app will display the weeks amonut）





Time & Date








